## Appendix B1

This database contains whole-rock geochemistry for 304 samples collected in the southern Buchans-Roberts Arm Belt, primarily from drillcore, but also includes several grab samples collected from outcrop. These samples were collected as part of deposit-level studies focusing on volcanogenic massive sulphide occurrences in the region. Samples were primarily collected from drillholes targeting known occurrences of base-metal mineralization and include representative samples of both mineralized and altered material as well as relatively unaltered country rock.

This database contains the results of major, trace and rare-earth element analyses and includes sample location data in Universal Transverse Mercator (UTM) eastings and northings, provided in NAD 83 (Zone 21); brief sample descriptions can be found in Appendix A3.

Samples were prepared at the Geological Survey of Newfoundland and Labrador’s (GSNL) Geochemistry Laboratory in St. John’s. Internal analyses carried out at the GSNL laboratory follow the methods outlined by Finch et al. (2018). Select samples were also submitted for external Instrumental Neutron Activation Analysis (INAA) at Bureau Veritas in Ontario and Cold Vapour-Atomic Absorption Spectrometry (CV-AAS) mercury analysis at Actlabs in Ancaster, Ontario. The following represents a brief summary of select analytical procedures utilized in determining the elements included in this study; a more detailed description can be found in Finch et al. (2018).

Major-elements plus select trace-elements were analyzed by inductively coupled plasma optical emission spectrometry (ICP-OES) following lithium tetraborate and metaborate fusion. Loss-on-ignition (LOI) and ferrous iron (FeO) values are also included with the major-elements and are determined through gravimetric and titration methods, respectively (c.f. Finch et al., 2018). Select trace-elements are also provided by ICP-OES following a four acid (HF-HCl-HNO3-HClO4) total digestion. The remaining trace-elements are determined by ICP-MS after a four acid (HF-HCl-HNO3-HClO4) total digestion. Fluoride values are provided by ion-selective electrode analysis following alkaline fusion. Silver analyses are done through ICP-OES following a nitric acid digestion. The procedure for external INAA analysis is summarized in Finch et al. (2018). Representative photographs of the rocks sampled for geochemistry are contained in Appendix B6. All databases are available in digital format (i.e. *.csv comma-separated value files) through the link provided in the Appendices section.

| List of abbreviations and codes |  |
| --- | --- |
| Abbreviation | Explanation |
| -99 | Sample not analyzed for that element |
| Avg | Average value |
| Dup. | Duplicate analysis |
| Fe2O3T | Total measured Iron |
| FeOT | total iron (II), calculated from total measured iron |
| GSNL | Geological Survey of Newfoundland and Labrador |
| Grav. | Gravimetric |
| CV-AAS | Cold Vapour Atomic Absorption Spectrophotometry |
| ICP-MS-FUS | Inductively Coupled Plasma Mass Spectrometry following lithium metaborate/tetraborate fusion |
| ICP-OES 4ACID | Inductively Coupled Plasma Optical Emission Spectrometry following HF-HCl-HNO3-HClO4 acid digestion |
| ICP-OES-FUS | Inductively Coupled Plasma Optical Emission Spectrometry following lithium metaborate/tetraborate fusion |
| ICP-OES-HNO3 | Inductively Coupled Plasma Optical Emission Spectrometry following nitric acid digestion |
| INAA | Instrumental Neutron Activation Analysis |
| ISE | Ion-selective Electrode |
| CRM | Certified Reference Material |
| DL | Detection Limit |
| LCL | Lower control limit |
| LOI | Loss on Ignition |
| N/A | Not available |
| pct | Percent |
| ppb | Parts per billion |
| ppm | Parts per million |
| Rec_Val | Recommended value |
| Ref | Reference |
| Std. | Standard |
| UCL | Upper control limit |
| wt_pct | Weight percent |

| List of abbreviations for CRM's |  |
| --- | --- |
| Abbreviation | Explanation |
| AGV-1 | Andesite, Lake County, OR (USGS) |
| BHVO-1 | Basalt, Hawaii (USGS) |
| BIR-1 | Basalt, Iceland (USGS) |
| CH-2 | CCRMP CH-2 Gold Ore |
| G-2 | Granite, Bradford RI (USGS) |
| MAG-1 | Gray-brown clayey mud, Gulf of Maine (USGS) |
| MP-1a | MP-1a ore |
| QLO-1 | Quartz Latite, Oregon (USGS) |
| RGM-1 | Rhyolite, California (USGS) |
| SDC-1 | Mica Schist, Washington DC (USGS) |
| STM-1 | Peralkaline nepheline syenite, Oregon (USGS) |
| SY-4 | Diorite gneiss, Brudendell Township ON (NRCAN) |
| SY-5 | Syenite, Township of Admaston/Bromley, ON (NRCAN). |
| WGB-1 | WGB-1: Gabbro, Wellgreen Complex YT (NRCAN) |