## Appendix B5

This appendix contains a compilation of all spectral data collected during this study, which includes outcrop, drillcore and hand sample measurements. The original spectral files, along with spreadsheets containing the mineral identification results and calculated scalar variables, as determined by the spectral software The Spectral Geologist (TSG; version 8.1.0.8), are included. These spectral measurements were collected using either an ASD TerraSpec Pro or TerraSpec Halo spectrometer.

Drillcore database:

| ID | Unique ID number |
| --- | --- |
| Year | Year spectra was collected |
| Prospect | Name of related mineral prospect |
| DDH | Associated drillhole collar number |
| Spec_Fm_m | Upper limit of spectral interval; in meters |
| Spec_To_m | Lower limit of spectral interval; in meters |
| Spec_Int_m | Spectral interval |
| Spec_Num | Original spectra sample number |
| Instrument | Instrument used to collect spectral data |
| Spec_Num_Halo | Spectral number assigned by Halo |
| Spec_Num_TSG | Spectral number exported from TSG |
| TSG_Min1 sTSAS | Mineral 1 identified by TSG |
| TSG_Wt1 sTSAS | Estimated mineral proportion for mineral 1 by TSG |
| TSG_Min2 sTSAS | Mineral 2 identified by TSG |
| TSG_Wt2 sTSAS | Estimated mineral proportion for mineral 2 by TSG |
| TSG_Error sTSAS | Error, or goodness-of-fit, of spectra to reference library spectra; smaller numbers = better match |
| TSG_Depth_1400 | Calculated spectral depth of hull quotient 1400 nm feature |
| TSG_Depth_1900 | Calculated spectral depth of hull quotient 1900 nm feature |
| TSG_Width_1900 | Calculated width of hull quotient 1900 nm feature |
| TSG_Depth_2200 | Calculated spectral depth of hull quotient 2200 nm feature |
| TSG_Width_2200 | Calculated width of hull quotient 2200 nm feature |
| TSG_Depth_2250 | Calculated spectral depth of hull quotient 2250 nm feature |
| TSG_Fe_OH | Position of spectral minimum at 2245 nm +/- 10nm |
| TSG_Mg_OH | Position of spectral minimum at 2330 nm +/- 10nm |
| TSG_WhtMica_Comp | Position of spectral minimum at 2204 nm +/- 24nm |
| TSG_ISM_H2O | White mica spectral maturity; depth of 2200 nm feature/depth of 1900 nm feature |

Field sample database:

| ID | Unique ID number |
| --- | --- |
| Prospect | Name of related mineral prospect |
| UTM_E | UTM easting |
| UTM_N | UTM northing |
| Datum | Datum |
| Zone | UTM Zone |
| Spec_Num_Halo | Spectral number assigned by Halo |
| TSG_Min1 sTSAS | Mineral 1 identified by TSG |
| TSG_Wt1 sTSAS | Estimated mineral proportion for mineral 1 by TSG |
| TSG_Min2 sTSAS | Mineral 2 identified by TSG |
| TSG_Wt2 sTSAS | Estimated mineral proportion for mineral 2 by TSG |
| TSG_Error sTSAS | Error, or goodness-of-fit, of spectra to reference library spectra; smaller numbers = better match |
| TSG_Depth_1400 | Calculated spectral depth of hull quotient 1400 nm feature |
| TSG_Depth_1900 | Calculated spectral depth of hull quotient 1900 nm feature |
| TSG_Depth_2200 | Calculated spectral depth of hull quotient 2200 nm feature |
| TSG_Depth_2250 | Calculated spectral depth of hull quotient 2250 nm feature |
| TSG_Fe_OH | Position of spectral minimum at 2245 nm +/- 10nm |
| TSG_Mg_OH | Position of spectral minimum at 2330 nm +/- 10nm |
| TSG_WhtMica_Comp | Position of spectral minimum at 2204 nm +/- 24nm |
| TSG_ISM_H2O | White mica spectral maturity; depth of 2200 nm feature/depth of 1900 nm feature |

Hand sample database:

| ID | Unique ID number |
| --- | --- |
| Year | Year spectral collected |
| SampleNum | Sample number |
| Prospect/Area | Name of related mineral prospect |
| UTMEast | UTM easting |
| UTMNorth | UTM northing |
| Datum | Datum |
| UTMZone | UTM Zone |
| DDH_StationID | Associated drillhole collar number or station ID |
| From_m | Upper limit of spectral interval; in meters |
| To_m | Lower limit of spectral interval; in meters |
| Interval_m | Spectral interval |
| Spectra_Num | Original spectra sample number |
| TSG_Min1 sTSAS | Mineral 1 identified by TSG |
| TSG_Wt1 sTSAS | Estimated mineral proportion for mineral 1 by TSG |
| TSG_Min2 sTSAS | Mineral 2 identified by TSG |
| TSG_Wt2 sTSAS | Estimated mineral proportion for mineral 2 by TSG |
| TSG_Error sTSAS | Error, or goodness-of-fit of spectra to reference library spectra; smaller numbers = better match |
| TSG_Depth_1400 | Calculated spectral depth of hull quotient 1400 nm feature |
| TSG_Depth_1900 | Calculated spectral depth of hull quotient 1900 nm feature |
| TSG_Depth_2200 | Calculated spectral depth of hull quotient 2200 nm feature |
| TSG_Depth_2250 | Calculated spectral depth of hull quotient 2250 nm feature |
| TSG_Fe_OH | Position of spectral minimum at 2245 nm +/- 10nm |
| TSG_Mg_OH | Position of spectral minimum at 2330 nm +/- 10nm |
| TSG_WhtMica_Comp | Position of spectral minimum at 2204 nm +/- 24nm |
| TSG_ISM_H2O | White mica spectral maturity; depth of 2200 nm feature/depth of 1900 nm feature |