1.0	Purpose

The purpose of this procedure is to detail the steps for testing water for fluoride.

2.0	Background – N/A

3.0	Procedure

- Perform a fluoride test or take fluoride reading from the automated equipment.

a.	If below or equal to 1.5 mg/L, go to step 2

b.	If above 1.5 mg/L, go to step 3

- Fluoride is below or equal to 1.5 mg/L – adjust fluoride metering pump to operate at between 0.9 to 1.4 mg/L and resume normal operation.

- Do a fluoride test to confirm results:

a.	 if below or equal to 1.5 mg/L go to step 4

b.	 if above 1.5 mg/L go to step 5

- Repeat fluoride test again:

a.	if below or equal to 1.5 mg/L go to step 2

b.	if above 1.5 mg/L go to step 5

Note: Must have 2 consecutive below or equal to 1.5 mg/L test results to return to step 2

- Fluoride at above 1.5 mg/L:

a.	advise supervisor who will contact appropriate personnel

b.	adjust fluoride metering pump to operate at between 0.9 to 1.4 mg/L

Town Manager/Clerk:  				Signature:				Date:

Perform a Fluoride test

Yes

Advise Supervisor -

Adjust fluoride metering pump to operate at between 0.9 to 1.4 mg/L

No

Yes

Is fluoride <= 1.5 mg/L?

No

Yes

Perform a Fluoride test to confirm result

Is fluoride <= 1.5 mg/L?

Perform a Fluoride test

No

Adjust fluoride pump to operate at between 0.9 to 1.4 mg/L -

Resume normal operation

Is fluoride <= 1.5 mg/L?