1.0	Purpose

The purpose of this procedure is to detail the steps for collecting water samples for microbiological analysis.

2.0	Background – N/A

3.0	Procedure

- Prior to commencing this procedure, the user shall put on surgical gloves to prevent contamination of the water samples.

- Remove the faucet aerator screen and burn off faucet tip area, for at least 20 seconds. Allow the water to flow for about 5 minutes prior to collecting the water sample.

- Fill bottle to indicator line. DO NOT FILL THE BOTTLE COMPLETELY. Only use the special sterile bottles provided for collecting the water samples.

- Replace the cap securely, label the bottle, and place the filled-up bottle in plastic bag and in carrier cooler.

- Complete the appropriate forms provided.

- Deliver all filled bottles and data forms to the appropriate government agency.

NOTES:

- Do not open the bottle until ready to collect sample

- Do not remove tablet from bottle

- Do not rinse out the bottle

- Do not touch the inside of the screw-cap or the mouth of the bottle

Town Manager/Clerk:  				Signature:				Date: