NL Master Specification Guide

for Public Funded Buildings
Re-Issued 2017/05/31
Section 12 21 26 – Black-out Blinds
Page 2 of 2

Part 1 GENERAL

1.1 RELATED SECTIONS

.1 Section 01 33 00 – Submittal Procedures.
.2 Section 01 74 21 – Construction/Demolition Waste Management and Disposal.
.3 Section 06 10 53 – Miscellaneous Rough Carpentry. 

.4 Section 12 21 16 – Vertical Louvre Blinds.
1.2 REFERENCES

.1 National Fire Protection Association (NFPA)

.1 NFPA 701: Standard Methods of Fire Tests for Flame Resistant Textiles and Films.
1.3 SUBMITTALS
.1 Indicate fabric dimensions in relation to existing window openings and operating hardware.
.2 Submit one representative working sample of each type horizontal louvre.

.3 Submit duplicate samples of manufacturer's standard colours for selection by Owner.

.4 After approval samples will be returned for incorporation into the Work.

1.4 WARRANTY


.1 Provide a written guarantee stating the black-out blinds will be free from manufacturer’s defects for a period of five (5) years from the date of Substantial Completion. 

.2 Contractor to provide a two (2) year warranty from the date of substantial completion against defects in workmanship.

Part 2 PRODUCTS
2.1 Materials and Fabrication
.1 Pull down shade of tearproof, opaque black.  Steel stays keep fabric flat and smooth in side channels.
.2 Shade mounted on one-piece heavy duty steel roller, not less than 38 mm diameter.
.3 Provide steel headbox at top of opening equipped with hinge cover for access.
.4 Steel side channels fully enclose edges of fabric. 

.5 Metal draw bar mounted on shade interlocks with steel sill channel by means of heavy duty automatic spring latch.

.6 Mounting: between window jambs. Verify all site dimensions.

Part 3 EXECUTION
3.1 INSTALLATION

.1 Install hardware using non-corrosive metal toggle bolts concealed in final assembly.
.2 Install square, plumb, true to line with operable parts adjusted for correct function.
.3 Install in accordance with reviewed shop drawings.
3.2 SCHEDULE

.1 Install blinds for all windows as indicated.

3.3 COMMISSIONING
.1 Instruct Owner in operation, adjustments, cleaning and maintenance.

END OF SECTION
Ver. 10/22

