NL Master Specification Guide

for Public Funded Buildings
Re-Issued 2020/08/28
Section 12 22 00 – Curtains and Drapes
Page 2 of 4

Part 1 GENERAL
1.1 RELATED SECTIONS

.1 Section 01 33 00 - Submittal Procedures.
.2 Section 01 43 39 – Mock Up Requirements.
.3 Section 01 78 00 - Closeout Submittals.


1.2 REFERENCE

.1 National Fire Protection Association (NFPA)

.1 NFPA 701, Fire Tests or Flame-Resistant Textile and Films.

.2 Underwriters Laboratories (UL)

.1 UL214, Flame-Propagation of Fabrics and films.

1.3 SUBMITTALS
.1 Submit working sample of Drapery Hardware.

.2 Submit flame resistance Test Report of Drapery Fabric.

.3 Drapery Fabric: submit a range of three samples, 900 mm x 900 mm or larger to match the fabric quality, weight, pattern and colour specified.

1.4 CLOSEOUT SUBMITTALS

.1 Provide operation and maintenance data for incorporation into manual specified in Sections 01 78 00 - Closeout Submittals. 

.2 Provide laundering and dry cleaning instructions for fabrics requiring special care.  Furnish separate instruction sheet for each material supplied.  For fabrics which are not permanently or inherently flame resistant, furnish instruction to include frequency and process required or retreating the fabric to renew the effectiveness of the flame treatment.  In lieu of instruction sheets, provide instructions on small, permanent labels affixed to back of the heading of each panel.

1.5 MOCK-UPS

.1 Construct mock up on accordance with Section 01 43 39 – Mock Up Requirements. 

.2 Provide one full size window installation including hardware.  Install the finished drapery on a traverse rod at the location agreed to by Owner.

Part 2 PRODUCTS
2.1 MATERIALS

.1 Drapery fabric: provide fabric manufactured from man made fibers.  Fabric physical characteristics shall be as follows:

.1 Pattern: woven

.2 Weave: closed

.3 Texture: Twill

.4 Color: as selected by Architect.

.5 Weight: approximately .66 lbs per lin. yard.

.6 Color fastness to light: AATCC Standard L5.

.2 Flame Resistance: Drapery fabric shall pass the small and large scale test of UL214 or NFPA 701.  Treatment to enhance flame resistance shall be permanent type.

.3 Sewing Thread: preshrunk mercerized cotton (50/3).

.4 Heading Hooks: slip-in type, chromium plated steel of a size adequate to hold the heading upright.  Provide one hook for each pleat.

.5 Drapery Hardware: Traverse wall mounted rods of heavy-duty type.  Traverse rods or tracks shall be manually operated, center close two-way.  Rods and tracks shall be cold-rolled, commercial quality steel minimum 0.75 mm thick or extruded aluminum minimum 1.27 mm thick.  Rod and track cross section width and depth shall be sufficient to carry the drapery without sagging.  Track configuration (number of channels) shall be such so as to permit drapery operation as specified or indicated.  Finish steel components with a white baked enamel, vinyl, or epoxy coating as standard with the manufacturer.  Provide smooth and non-sticking sliding surfaces.  Provide one-piece rod and track up to 4875 mm long.  Provide steel brackets and intermediate supports.  Provide one manufacturer’s design throughout. 

.6 Traverse cord: Size No. 4 with fiberglass center.  Provide cord, tassel with lead weight center and plastic coating, tension pulley, metal tube type, with mounting bracket, helical spring and ball bearing.  Finish colour white.  

.7 Fasteners: Zinc or cadmium plated.

2.2 FABRICATION

.1 Panels: prior to cutting and fabrication, field measure each drapery location paying particular attention to field conditions affecting the work made from full or half widths of fabric to five a minimum of 250 percent fullness.  250 percent fullness is defined as 2 1/1 times the rod width plus overlaps and returns.  Provide conventional french pleats.  Draperies shall be sill length.  Sill length draperies shall hang 13 mm above window sills and heating-air conditioning units.  Provide table-sized drapery panels with a plus or minus tolerance of 6 mm accurately laid-out before cutting.  Cut fabric to allow for pleats and for outside ends to return to the walls.  For traverse draperies, allow for a patterned fabrics to provide identical designs horizontally and vertically on each window within each room.  Where variations in length or placement of windows occur in a room, match patterns horizontally. When fabricating to panels from fabrics which require special methods or instructions, conform to the workroom instructions provided by the fabric manufacturer.  Sew seams and hams using a firm interlocking stitch as a stitch rate per millimeter appropriate to fabric being sewn.  Sew with enough slack present so that thread shrinkage due to laundering and dry cleaning will not pucker seams and hems Do not expose seam and hem raw edges. 

.2 Headings: pleat evenly to required widths.  Make headings 100 mm high with triple french pleats, and double fold.  Include permanent finish stiffener of buckram. Crinoline, or pellon across entire heading.  Paper stiffening is not acceptable.  Machine stitch pleats for a depth of at least 95 mm.  Do not use horizontal stitching across the width of the heading.

.3 Seams: join widths by serging, overlock, and safety stitch.  Retain selvage when practical.

.4 Hems: double fold hems (top and bottom) and blink stitch so as not to show on the panel face.  Make side seams 38 mm wide and bottom seams 100 mm deep with weighs sewn 13 mm above hem bottom.  Provide weights at corners and each vertical seam.  When lining is attached to the drapery fabric, single fold heading is acceptable, however, double fold the bottom hem.

.5 Valances:  Rod-hung, fabricated in the same manner as draperies and of same material.

Part 3 EXECUTION

3.1 EXAMINATION

.1 Ensure that work of other trades and cleaning operations are completed.

.2 Install draperies in rooms and areas indicated. Include all material indicated, specified or necessary for a complete finished installation. Contractor shall be responsible for the required quantities of draperies and hardware.

.3 Install hardware in accordance with the manufacturer’s printed instructions and as specified herein. Install wall rods with end brackets and provide intermediate support brackets 600 mm from each end with additional intermediate support brackets spaced not more than 1200 mm apart. Provide fasteners for installation as follows:


Fasteners




Structural Material

.1 Wood or sheet metal screw
Wood

.2 Self tapping screws


Metal

.3 Case hardened, self-tapping



.4 Sheet metal screw


Sheet Metal

.5 Screw or bolt in expansion





.6 Shield




Solid Masonry

.7 Toggle or molly bolt


Hollow masonry, wall board, plaster

.4 Install draperies with a minimum clearance of 6 mm between the ceiling and top of drapery heading. Insert heading hooks at rear of each pleat, placed to obtain the clearance specified. Press well before hanging except fiberglass. Dress-down and adjust hung draperies to provide best form and appearance. Traverse draperies shall operate smoothly and easily over the full range of travel. Remove incorrectly sized drapery and remake to correct size. Remove damaged, spotted or otherwise defective fabric and repair to original state or replace with new material. 

END OF SECTION
Ver. 10/22

