NL Master Specification Guide

for Public Funded Buildings
Re-Issued 2021/05/31
Section 26 27 73 - Door Chimes
Page  
 of 2

Part 1 GENERAL

1.1 RELATED SECTIONS
.1 Section 01 33 00 - Submittal Procedures.
.2 Section 01 91 13.13 - Commissioning (Cx) Requirements.
.3 Section 26 05 00 - Common Work Results - Electrical.

1.2 REFERENCES

.1 Canadian Standards Association (CSA)
.2 Underwriters' Laboratories of Canada (ULC)

Part 2 PRODUCTS
2.1 EQUIPMENT
.1 Door bells: single or double tone, 50 mm diameter gong, buzzer or chimes, ac 24 V, 3 A, indoor or outdoor weatherproof, mounting surface, colour white, sound level 10 dB at 3.0 m, ULC listed as indicated.
.2 Class II transformers: 24 V, 3 A secondary. 120 V, 60 Hz primary, with flush two gang mounting, CSA listed, thermal protected.

.3 Pushbuttons: low voltage, lighted style for surface mounting.  

.4 Low voltage wiring: type LVT or as indicated.

Part 3 EXECUTION
3.1 INSTALLATION
.1 Attach components to box where indicated with screws and expanded shields.  
.2 Install wiring.

.3 Remove packing material and construction dirt around plunger.

3.2 FIELD QUALITY CONTROL
.1 Perform tests in accordance with Section 26 05 00 – Common Work Results - Electrical and Section 01 91 13.13 – Commissioning (Cx) Requirements. 
.2 Test system for operation and sound level.

END OF SECTION

Ver. 10/22

