NL Master Specification Guide

for Public Funded Buildings

Section 26 28 23 – Disconnect Switches – Fused
Re-Issued 2021/05/31
 and Non-Fused
Page 1 of 2

Part 1 General

1.1 Related Sections
.1 Section 01 33 00 - Submittal Procedures.
.2 Section 01 91 13.13 – Commissioning (Cx) Requirements.
.3 Section 26 05 00 – Common Work Results - Electrical.
Part 2 Products
2.1 Disconnect Switches
.1  Fusible and non-fusible, disconnect switch in CSA Enclosure type 1, size as indicated.
.2 Provision for padlocking in on-off switch position by three locks.

.3 Mechanically interlocked door to prevent opening when handle in ON position.

.4 Fuses: size as indicated, to Section 26 28 13.01 - Fuses - Low Voltage.

.5 Fuseholders: suitable without adaptors, for type and size of fuse indicated.

.6 Quick-make, quick-break action.

.7 ON-OFF switch position indication on switch enclosure cover.

2.2 Equipment Identification
.1 Provide equipment identification in accordance with Section 26 05 00 – Common Work Results - Electrical.
.2 Indicate name of load controlled on size 4 nameplate.

Part 3 Execution
3.1 Installation
.1 Install disconnect switches complete with fuses as indicated.
END OF SECTION

Ver. 10/22

