NL Master Specification Guide

for Public Funded Buildings
Issued 2008/03/18                  Section 26 42 00 – Cathodic Protection
Page 1 of 2

Part 1 General

1.1 Related Sections
.1 Section 01 33 00 - Submittal Procedures.
.2 Section 26 05 00 - Common Work Results - Electrical.

1.2 References

.1 Canadian Gas Association (CGA)
.1 CGA Recommended Practice OCC-1, for the control of external corrosion on buried or submerged metallic piping systems.
.2 Canadian Standards Association (CSA)
.1 CSA C22.3 No.4- Control of Electrochemical Corrosion of Underground Metallic Structures.
.3 National Association of Corrosion Engineers (NACE)
1.3 SUBMITTALS
.1 Indicate location of anodes, test positions, connecting cables, wire splicing details, installation procedures, power supply, hardware and accessories.

1.4 Warranty
.1 Contractor hereby warrants that the cathodic protection will be effective in accordance with General Conditions, for a period of five years.
Part 2 Products
2.1 Equipment
.1 Provide complete cathodic protection system as indicated, comprising galvanic or sacrificial anodes having a minimum design life, of 25 years: to CSA 22.3 No.4.
Part 3 Execution
3.1 Installation

.1 Install system including bonding of system to structure as indicated.

.2 Locate test stations in weatherproof housings as indicated.

.3 Make joints and connections by means of thermit welding. Bond pipelines to structure.

END OF SECTION

Ver. 10/22

